!

! How to obtain a plot of emission lines from the

! calibration tables, e.g. neon.tbl, hear.tbl or thar.tbl

!

!

! 1- Convert the FITS table to MIDAS table

INDISK/FITS hear.mt hear.tbl

!

! 2- Create another two identical tables and

!    set all INTENSITY values to 0.

COPY/TABLE hear hearA

COMPUTE/TABL hearA :INTENSITY = 0

COPY/TABLE hear hearB

COMPUTE/TABLE hearB :INTENSITY = 0

! The first table is shifted -0.001 in WAVELENGTH

! The second table is shifted +0.001 in WAVELENGTH

COMPUTE/TABLE hearB :WAVE = :WAVE - 0.001

COMPUTE/TABLE hearB :WAVE = :WAVE + 0.001

! The final table is create as a merged of the

! original one and the two shifted tables with

! INTENSITY=0

MERGE/TABLE hear hearA hearB hear_final

! The final table is then sorte on incremental WAVELENGTH

SORT/TABLE hear_final :WAVE

! Convert table into FITS for use in other platforms

OUTDISK/FITS hear_final.tbl hear_final.mt

! Now we can plot the table connecting the points (forcing

! each emission line to go from 0 , to INTENSITY and back to 0

! therefore simulating an emission line

SET/GRAPH pmode=1  ! no MIDAS banner

CREATE/GRAP

PLOT/AXIS 3500,5000 -30,800

SET/GRAPH colour=2

OVERPLOT/TABLE hear_final :WAVE :INTENSITY ? 0

! Create a GIF

COPY/GRAPH postscript

$mv postcript hear_3500_5000.ps

$xv hear_3500_5000.ps

(rotate clockwise and safe it as GIF)